# Garden Tool Maintenance

This playbook outlines the steps required for properly cleaning, sharpening, and storing gardening tools. Following these guidelines will help maintain tool condition and extend their usage lifespan.

### Step 1: Gather Tools

Collect all gardening tools that require maintenance. This includes shovels, spades, pruners, hoes, and any other tools used in the garden.

### Step 2: Clean Tools

Remove soil and debris from the tools using a stiff brush. If necessary, wash the tools with water and a mild detergent, then dry them thoroughly to prevent rust.

### Step 3: Inspect Tools

Check each tool for signs of wear or damage such as loose handles, bent blades, or rust. Make a list of tools that need repair or replacement parts.

### Step 4: Sharpen Blades

Use a file or sharpening stone to sharpen the cutting edges of tools such as pruners, shears, and shovels. Always follow the original angle of the cutting edge for best results.

### Step 5: Oil Moving Parts

Apply lubricating oil to the moving parts of tools such as pruners, shears, and hoes to ensure smooth operation and prevent rust.

### Step 6: Handle Care

Sand any rough spots on wooden handles. Once smooth, apply linseed oil to help preserve the wood and prevent it from drying out and splintering.

### Step 7: Store Properly

Hang tools or place them in a dry, covered area to protect them from moisture and direct sunlight. Do not leave tools on the ground or in a pile where they can become damp.

## General Notes

### Frequency

Consider performing maintenance on your gardening tools at least once at the beginning and end of the gardening season.

### Safety

Wear protective gloves and safety eyewear when cleaning and sharpening tools to prevent injuries.

### Disposal

Safely dispose of any cloths or rags used with oil or sharp debris to prevent fire hazards or injury.